I want you to build a CFD app. You can get inspiration from the code located in ./code-for-inspiration/, specifically with regards to the GUI: how the shape can be drawn, the layout, the streamlines for visualization. The solver, however, should be built from scratch. Here are the specifications:

Physics: 2D, incompressible, viscous Navier–Stokes

Time treatment:

Stage 1: Use pseudo-transient time-stepping tactics to try to find a steady state quickly. Check convergence of residuals.If all flatten enough, report a steady solution found.

Stage 2: If Stage 1 does not convincingly settle, restart from the current field and switch to real physical-time integration. In other words, Start from the latest field from Stage 1 (warm-start), then run with a physical time step. Ignore an initial buffer window, then record the time evolution of the solution.

Build one transient-capable solver with a “fast steady-search mode” and a “physical transient mode”.

Geometry handling: immersed boundary with Brinkman penalization

Discretization: finite volume

Pressure–velocity coupling: projection / pressure-correction method

Linear solve: fast Poisson solver, ideally multigrid

BCs: left velocity inlet, right pressure outlet, top/bottom slip or symmetry, obstacle no-slip

Turbulence model: none

Benchmarks: You will create benchmarks to verify the solver implementation. These should be built so that they can be re-run seamlessly whenever a modification is made to the code. For all these test cases, the code needs to generate a verification report with all relevant details and plots.

1) Penalized plane Poiseuille or Couette flow, with the walls represented by Brinkman masks.  This is the best first wall test because the target solution is analytic. Measure L

L2 error of the velocity profile, and sensitivity to grid size and penalization parameters.

2. Steady flow past a circular cylinder at Re=40.

See \literature\Gautier-et-al-Reference-solution-flow-over-a-circular-cylinder.pdf for the details and how to measure the solver implementation is correct.